# Guidelines for The Roxtec Package in Aveva E3D Design

# Instructions for Modeling Cable- and Pipe transits utilizing the features of the Roxtec Package

Guideline Version: 1.0

Completion Date: TBD

Responsible department: TACS BA M&O

Status: Working/Unreleased

Contents

Guidelines for The Roxtec package in Aveva E3D Design	0

Instructions for Modeling Cable- and Pipe transits utilizing the features of the Roxtec Package	0

1 Pipe penetrations	2

Step 1 – Preparation.	2

Step 2 – Grouping Items.	2

Step 3 – Class Selection.	3

Step 4 – Type Selection.	4

Step 5 – Hole Manager.	5

Step 6 –Hole Association.	5

Step 7  –  Approving the Holes.	6

Step 8 – Completion.	7

2 Cable penetrations	9

Step 1 – Prepreation.	9

Step 2 – Creation of penetrations.	9

Step 3 – Class selection	10

Step 4 – Hole manager	11

Step 5 – Hole association	12

Step 6 – Approving the holes	12

Step 7  –  Completion	13

# 1 Pipe penetrations

Step 1 – Preparation. Add the relevant parts of the 3D model to the drawlist.

Step 2 – Grouping Items. Open the “Create penetration” form by clicking Pipe -> Create Penetration in the Penetrate group in the ribbon bar. Add the wall as penetrated item and the relevant pipe or pipes as penetrating elements. Make sure “Penetrations” is selected in the form. Click OK.

Step 3 – Class Selection. The form “Hole Management – Definition” will open. In the “Class” drop down gadget all defined Roxtec penetrations are available. Select the relevant one.

Step 4 – Type Selection. When a Roxtec class of penetrations is selected the “Type” drop down gadget will be populated with relevant types.

Step 5 – Hole Manager. Open the Hole Manager application by clicking Holes -> Hole Manager in the ribbon bar.

Step 6 – Hole Association. The Hole Association Manager form will open. Search for the holes, select them and click “Manage selected holes” near the bottom of the form.

Step 7 –  Approving the Holes. In the Hole Management form, select all holes and, as the relevant user, click “Request”. The link label “Approve” is now activated. Click it to approve the hole.

Step 8 – Completion. When “Approve” is clicked in the previous step the actual holes are created in the wall. The sleeves can also be seen graphically in the 3D view.

# 2 Cable penetrations

Step 1 – Prepreation. Add the relevant parts of the 3D model to the drawlist.

Step 2 – Creation of penetrations. Open the “Create penetration” form by clicking Cableway in the Penetrate group in the ribbon bar. Add the wall as penetrated item and the relevant cable tray or trays as penetrating elements. Make sure “Penetrations” is selected in the form. Click OK.

Step 3 – Class selection. The form “Hole Management – Definition” will open. In the “Class” drop down gadget all defined Roxtec penetrations are available. Select the relevant one.

Step 4 – Hole manager. Open the Hole Manager application by clicking Holes -> Hole Manager in the ribbon bar.

Step 5 – Hole association. The Hole Association Manager form will open. Search for the holes, select them and click “Manage selected holes” near the bottom of the form.

Step 6 – Approving the holes. In the Hole Management form, select all holes and, as the relevant user, click “Request”. The link label “Approve” is now activated. Click it, to approve the hole.

Step 7 – Completion. When “Approve” is clicked in the previous step the actual holes are created in the wall. The frame can also be seen graphically in the 3D view. This is also when the actual holes are created in the wall. When penetrations are created and approved within the 3D model the results is as below: